VUBug:  AN M68000 MONITOR PROGRAM

I.  Introduction.

VUBug is a primitive single user monitor program designed

to interface the user to the "Raymatic" M68000 development board

designed by Raymond Carr at Vanderbilt University in 1988.  The

monitor program itself was originally written by Edward M. Carter

also while at Vanderbilt in 1983.  This documentation relates to

the 1988 version of the program, about two thirds of which is new.

The new stuff was added by Russell G. Brown, then of Vanderbilt, but

lately of Cornell Computer Science.  It allows the user to perform

such operations as are necessary to develop and debug M68000

assembly programs in conjunction with a host computer running

an M68000 assembler which is capable of generating object code in

the Motorola S-record format.  Some functions provided by VUBUG

are load from host, dump memory, disassemble memory, and run at

address.  Feel free to distribute this source and documentation,

but make sure that the copyright and like information stays on it.

Also, please make sure that any modified version of this program

which you may distribute is obviously marked as being modified.

II.  Running VUBug

VUbug comes up running as soon as power is applied to the

system.  It asserts its presence by printing the following

message:

M68000 Monitor VUBug Version 4.0 27 April, 1988

!_

If there is anything wrong with the system RAM, it will instead

respond

RAM Error!!!!

Assuming that a RAM error doesn't happen, you can now begin

entering commands.

III.  Getting Started

The first thing that you will want to do is to load and

execute an assembly language program using VUBug.  The following

is an example interactive execution session:

1.   Turn on the system.  The VUBug boot message prints at the

terminal.

2.   Type 'e'.  VUBug responds "Terminal Mode:".

3.   Log into the host system.  Enter vi and type in the

following program:

org     $4400

move.l  #$00000004,d0

movea.l #$000019ca,a3

adda.w  d0,a3

move.l  a3,d0

trap    #$06

trap    #$0

end

4.   Write and quit from vi.  Assemble the program by

typing

mas68k -l <progname>.

where progname is the name of your sample program. 5.

Examine the listing file produced by mas68k to see the source

and object code together.  The listing file is called

<progname.l>.

6.   Type 'cat <progname.o>' but DO NOT! type a carriage return.

7.   type ctrl-L.  VUBug will exit terminal mode, feed a

carriage return to the host, and accept an Srec format object

file.

8.   You are now back in VUBug.  Type 'a 4400'.  You will see an

assembly listing which is equivalent to your source code,

except that it has values substituted for all labels.  There

will be a number of garbage lines following the program.

disregard these.

9.   Type 'g 4400'.  This is the command meaning "go at $4400".

VUBug responds with the following:

Program:

000019ce

!

This is approximately how all debugging sessions should

begin.  Once you have become more familiar with VUBug, you will

learn shortcuts and enhancements, but the commands already used

will serve for a start.

IV.  VUBug Commands

This section contains the full, alphabetic listing of the

commands provided in VUBug.  For information on how to get help

for commands while in VUBug, see the section on 'h'.

A - Assembly Listing

A provides a source code listing of memory locations

specified in one of the following ways:

A<cr>           List 20 instructions from last

examined address.

A xxxx          List 20 instructions from address

xxxx.

A xxxx,yyyy     List from xxxx to yyyy.

B - Set/Remove Breakpoints

B permits the setting and removing of breakpoints in a user

program.  B is invoked as follows:

B<cr>           List all breakpoints.

B+xxxx          Add a breakpoint at address xxxx.

B-xxxx          Remove a breakpoint at xxxx.

B#              Remove all breakpoints.

C - Copy Memory

C allows the copying of a data block from one location to

another.  It is called using:

C xxxx=yyyy,zzzz.

This copies memory from a block which starts at yyyy and

ends at zzzz to a block starting at xxxx.  NOTE:  the copy

routine works from bottom to top.  Therefore care must be

taken when copying between blocks that share address space.

D - Dump Memory

D is used to provide a hex dump of memory.  Memory is

printed out in lines consisting of an address, 16 hex

values, and the ascii representation of such of those

values as have printable characters.  The format for

calling D is as follows:

D<cr>           Dump 64 bytes from last examined

location.

D xxxx          Dump 64 bytes from xxxx.

D xxxx,yyyy     Dump bytes from xxxx to yyyy.

E - Enter terminal emulator mode

When E is entered, VUBug enters a passthrough mode in which

any key entered at the keyboard is passed through to the

host, and vice versa.  This is true for all characters

except for ctrl-X, which causes VUBug to exit terminal

emulator mode, and ctrl-L, which is the same as ctrl-X,

except that it causes VUBug to enter load mode.  E is

invoked as follows:

E<cr>

G - Start user program

G is the VUBug command to start a user program running.  A

user program which is properly designed to end will have as

its last executed line

trap    #$00

G is called in one of the following ways:

G<cr>           Go from start address in the last

executed L command.

<cr>            Same as G<cr>.

G xxxx          Go from address xxxx.

H - Print help messages.

H is used to print a simple message (consisting mostly of

instruction syntax) to the terminal.  It is called with:

H<cr>           Prints a list of commands for

which help is available.

Hx              Prints the long help message for

command x.  If x is nonexistant,

Hx acts like H<cr>.

L - Load user program from host

L causes VUBug to accept an S-record format object file

from the host.  It is called in one of the following ways:

L<cr>           Load from host with no offset.

ctrl-L          Same as L<cr> but invoked from

emulator mode.

L xxxx          Load from host with offset xxxx.

M - Enter memory update mode

M invokes a submode of VUBug in which modifications to

memory locations can be made.  M is invoked as follows:

M<cr>           Start memory mode.

M xxxx          Start memory mode at xxxx.

Memory mode subcommands are the following:

.xxxx           Set pointer to location xxxx.

=xx             Update current location to xx.

,xx             Increment pointer, then update.

+               Increment pointer.

-               Decrement pointer

<cr>            Print current pointer and value.

Typing any other character in memory update mode causes an

exit back into VUBug proper.

P - Prototype commands in RAM

P allows the user to create new command subroutines in RAM

and call them without having to burn new ROMs.  This is

significant in that ordinarily user programs cannot make

use of the interrupt routines and other subroutines

provided in VUBug.  P is set up to allow a user routine to

make use of these subroutines.  It is invoked as follows:

Px xxxx         Load address xxxx for prototype

command x.

Px<cr>          Execute prototype command x.

Note that x must be a 1, 2, or 3.

R - Enter register examine/update mode

R invokes a submode of VUBug in which registers may be

examined and changed.  It is called in the following way:

R<cr>           Start register mode.

R xx            Start mode at register xx,

where xx is one of:

SR/_sr - status register,

PC/_pc - program counter,

dx - data register 0 - 7,

ax - address reg. 0 - 7.

Register examine/update commands are as follows:

.xx             Set pointer to register xx.

=xxxxxxxx       Set current register to xxxxxxxx.

<cr>            Print all registers.

Any other character entered in register submode causes an

exit to VUBug.

S - Single step mode

In single step mode, a Go command causes VUBug to execute a

single instruction and then return to the prompt, allowing

registers to be examined and changed between instructions.

S is called in one of two ways:

S+              Turn on single step mode.

S-              Turn off single step mode.

T - Trace mode

In trace mode, a Go command causes VUBug to execute a

program, printing out the PC after each instruction.  This

permits tracing of a program's path.  It is called as

follows:

T+              Turn on trace mode.

T-              Turn off trace mode. V.

U - Upload user program from terminal

U allows VUBug to upload an S-record format object file

from the terminal.  This allows the user to connect the

68000 board to a microcomputer, using it as both terminal

and host.  Upload is invoked with one of the following

commands:

U<cr>           Upload from terminal with no offset.

U xxxx          Upload from terminal with offset xxxx.

V.   Troubleshooting

There is not a whole lot that can go wrong with VUBug.

Most of the possible difficulties actually have their roots in

hardware.  If a RAM error is indicated at boot-up time, then one

of the system RAM chips needs replacing.  If no boot-up message

is printed, then something is wrong with one of the following:

the terminal, the terminal cable, the ACIAs on the development

board, the system RAM, or the VUBug ROM.

If an error is found in VUBug's performance, the thing to

do is to document carefully when and where the bug appears, and

report the problem to the nearest digital electronics professor.

VI.  Caveats

A number of things should be taken into account when using

VUBug.  Most of these have to do with the memory map.  Number

one is that hitting the reset button invokes the RAM test program

which CLEARS THE RAM!  For this reason, ctrl-C should be used to

recover control from a lost program.

The memory map for the board looks like this:

0000 - 3FFF     System ROM

4000 - 43FF     System RAM

4400 - 7FFF     User RAM

8000 - 803E     PI/T (68230) registers EVEN ONLY!

A000            Load Port Status Register

A002            Load Port Data Register

C000            Terminal Port Status Register

C002            Terminal Port Data Register

Note that the PI/T and ACIA registers are on even only

locations.  Attempting to write to a peripheral chip on an odd

location will lock up the system.  Similarly, writing to system

RAM can cause unexpected results, up to and including locking

up the system.

VII. Conclusion

VUBug 4.0, mas68k, and the Raymatic development board form

an effective development environment for M68000 assembly

language programs.  While they cannot substitute for

professional development/analysis systems, it is nevertheless

possible to complete considerable software endeavors with a

reasonable minimum of difficulty.